Logical Address  IP Address
Physical Address  MAC Address

# ARP (Address Resolution Protocol)

10.10.10.0/24  IP address of the entire network
10.10.10.1  IP address of PC-0
10.10.10.2  IP address of PC-1
10.10.10.3  IP address of PC-2
10.10.10.4  IP address of PC-3
10.10.10.5  IP address of PC-4

For a PC to communicate to another PC within a local network, only IP address of that particular PC is not sufficient. Particular PC’s MAC address is also required.

Eg: PC1 wants to communicate with PC3.

## Checking Local Cache table

We will check in the cache table of PC1 whether PC3’s MAC address is already available/not.

Cache table  PC 3’s MAC address is available. Without generating any ARP request ,PC1 can send the ICP packets directly with PC3.

## MAC Address not available

Cache table  PC 3’s MAC address is not available. So PC1 will send the ARP request to the PC whose IP is 10.10.10.4 to the switch. 
ARP packet  green envelope
ICMP packet    red envelope

## Inside ARP packet

What is inside ARP packet ??

Source MAC Address (PC-1)  0001.9747.1BA9 
Source IP Address (PC-1)  10.10.10.2

Destination MAC Address (PC-3)  FFFF.FFFF.FFFF  
[ Initially PC-3’s MAC address is not known ]
Source IP Address (PC-3)  10.10.10.4

## ARP packet to switch

Then that  ARP packet  green envelope is sent to the switch

ARP packet  green envelope reached the switch.

## (Request) Broadcasting the ARP packets to the entire network

Then that switch will broadcast the ARP packet  green envelope  to the entire PC’s in the network except PC-3 (since ARP packet was sent from PC-3)

## Response From the entire network

X marks indicates that 10.10.10.4 does not belong to PC-0, PC-2 and PC-4.
10.10.10.4 belongs to PC-3. So there is no X mark.

## Opening the message in PC-3

PC-3’s MAC address		PC-1’s MAC address

Source IP (PC-3)  10.10.10.4
Destination IP (PC-1)  10.10.10.2

## Sending the reply message from PC-3 to PC-1

## PC-1 knows the MAC address of PC-3

- Indicates that PC-1 got the MAC address of PC-3 and PC-1 is ready to send the ICMP packets.

## PC-1 to PC-3

PC-1 sends the ICMP packet    red envelope directly to PC-3.